Pratham Paleriya

21165

A4 Batch

EXPERIMENT NO : 13

TITLE :

Develop a Real time application like smart home with following requirements: When the user enters into the house the required appliances like fan, light should be switched ON. Appliances should also get controlled remotely by a suitable web interface. The objective of this application is that student should construct complete Smart application in groups.

CODE:

HTML Code

<html>

<title>Home Automation Demo</title>

<body bgcolor=#C7C7FF>

<center>

<h1 style="color:#FF0000">MicroEmbedded Technologies</h1> <h2 style="color:#00AF00">Home Appliance Control</h2> <form action="index.php">

Door Latch &nbsp<button type="submit" name="op" value="1"> OFF</button> &nbsp &nbsp <button type="submit" name="op" value="2"> ON</button> <br><br>

Light1 &nbsp<button type="submit" name="op" value="3"> OFF</button> &nbsp &nbsp <button type="submit" name="op" value="4"> ON</button> <br><br>

Fan1 &nbsp<button type="submit" name="op" value="5"> OFF</button> &nbsp &nbsp <button type="submit" name="op" value="6"> ON</button> <br><br>

Light2 &nbsp<button type="submit" name="op" value="7"> OFF</button> &nbsp &nbsp <button type="submit" name="op" value="8"> ON</button> <br><br>

Fan2 &nbsp<button type="submit" name="op" value="9"> OFF</button> &nbsp &nbsp <button type="submit" name="op" value="10"> ON</button> <br><br>

Light3 &nbsp<button type="submit" name="op" value="11"> OFF</button> &nbsp &nbsp <button type="submit" name="op" value="12"> ON</button> <br><br>

Fan3 &nbsp<button type="submit" name="op" value="13"> OFF</button> &nbsp &nbsp <button type="submit" name="op" value="14"> ON</button> <br><br>

Appliance 1 &nbsp<button type="submit" name="op" value="15">

OFF</button> &nbsp &nbsp <button type="submit" name="op" value="16">

ON</button> <br><br>

Appliance 2 &nbsp<button type="submit" name="op" value="17"> OFF</button> &nbsp &nbsp <button type="submit" name="op" value="18"> ON</button> <br><br>

<!--input type="submit" value="Submit"-->

</form>

</center>

</body>

</html>

PHP Code

<!DOCTYPE html>

<?php

$op = $_GET['op'];

shell_exec("/usr/local/bin/gpio -g mode 6 out");	#FAN1

shell_exec("/usr/local/bin/gpio -g mode 16 out"); #FAN2

shell_exec("/usr/local/bin/gpio -g mode 21 out");

shell_exec("/usr/local/bin/gpio -g mode 22 out"); #Mains2

shell_exec("/usr/local/bin/gpio -g mode 23 out"); #Light1

shell_exec("/usr/local/bin/gpio -g mode 24 out");

shell_exec("/usr/local/bin/gpio -g mode 25 out"); #Light3

shell_exec("/usr/local/bin/gpio -g mode 5 out");

shell_exec("/usr/local/bin/gpio -g mode 27 out"); #Mains1

#Door Latch

#Light2

#FAN3

switch($op){

case 1: shell_exec("/usr/local/bin/gpio -g write 21 0");#Door Latch OFF break;

case 2: shell_exec("/usr/local/bin/gpio -g write 21 1");#Door Latch ON break;

case 3: shell_exec("/usr/local/bin/gpio -g write 23 0");#Light 1 break;

case 4: shell_exec("/usr/local/bin/gpio -g write 23 1"); break;

case 5: shell_exec("/usr/local/bin/gpio -g write 6 0");#FAN1 break;

case 6: shell_exec("/usr/local/bin/gpio -g write 6 1"); break;

case 7: shell_exec("/usr/local/bin/gpio -g write 24 0");#Light2 break;

case 8: shell_exec("/usr/local/bin/gpio -g write 24 1"); break;

case 9: shell_exec("/usr/local/bin/gpio -g write 16 0");#FAN2 break;

case 10: shell_exec("/usr/local/bin/gpio -g write 16 1"); break;

case 11: shell_exec("/usr/local/bin/gpio -g write 25 0");#Light3 break;

case 12: shell_exec("/usr/local/bin/gpio -g write 25 1"); break;

case 13: shell_exec("/usr/local/bin/gpio -g write 5 0");#FAN3 break;

case 14: shell_exec("/usr/local/bin/gpio -g write 5 1"); break;

case 15: shell_exec("/usr/local/bin/gpio -g write 27 0");#Mains1 break;

case 16: shell_exec("/usr/local/bin/gpio -g write 27 1"); break;

case 17: shell_exec("/usr/local/bin/gpio -g write 22 0");#Mains2

break;

case 18: shell_exec("/usr/local/bin/gpio -g write 22 1"); break;

default:shell_exec("/usr/local/bin/gpio -g write 21 0"); shell_exec("/usr/local/bin/gpio -g write 22 0"); shell_exec("/usr/local/bin/gpio -g write 23 0"); shell_exec("/usr/local/bin/gpio -g write 24 0");

shell_exec("/usr/local/bin/gpio -g write 25 0");

shell_exec("/usr/local/bin/gpio -g write 26 0");

shell_exec("/usr/local/bin/gpio -g write 27 0");

shell_exec("/usr/local/bin/gpio -g write 5 0");

shell_exec("/usr/local/bin/gpio -g write 16 0");

}

include("page.html");

?>

Python Code

import time

import RPi.GPIO as GPIO

RUNNING = True

HIGH = 1

LOW =0

PIRPin = 4

DOORpin = 21

LIGHT1pin = 23

LIGHT2pin = 24

LIGHT3pin = 25

| FAN1pin | = 6 |
| --- | --- |
| FAN2pin | = 16 |
| FAN3pin | = 5 |
| Mains1pin = 27 |  |
| Mains2pin = 22 |  |

def InitSystem():

GPIO.setmode(GPIO.BCM)

GPIO.setup(PIRPin,GPIO.IN,pull_up_down=GPIO.PUD_UP)

GPIO.setup(DOORpin,GPIO.OUT)

GPIO.setup(LIGHT1pin,GPIO.OUT)

GPIO.setup(LIGHT2pin,GPIO.OUT)

GPIO.setup(LIGHT3pin,GPIO.OUT)

GPIO.setup(FAN1pin,GPIO.OUT)

GPIO.setup(FAN2pin,GPIO.OUT)

GPIO.setup(FAN3pin,GPIO.OUT)

GPIO.setup(Mains1pin,GPIO.OUT)

GPIO.setup(Mains2pin,GPIO.OUT)

return

def DetectPerson():

#while True:

input_state = GPIO.input(PIRPin)

time.sleep(0.3)

if input_state == 0:

return LOW

else:

return HIGH

try:

| print "\n\n | Home Automation Testing\n\n" |
| --- | --- |
| print "----------------------------------------------- | \n" |
| InitSystem() |  |
| count =0; |  |
| count_flag =0 |  |
| door_flag =0 |  |
| elapsed =0 |  |

start = time.time()

while RUNNING:

state = DetectPerson()

if state == HIGH:

if count_flag == 1:

count_flag =0

count+=1

print "Person Detected\n"

else:

count_flag = 1

#print "Waiting for Next Person. Time elapsed %d\n" %elapsed if count == 0:

GPIO.output(DOORpin,0)

GPIO.output(LIGHT1pin,0)

GPIO.output(LIGHT2pin,0)

GPIO.output(LIGHT3pin,0)

GPIO.output(FAN1pin,0)

GPIO.output(FAN2pin,0)

GPIO.output(FAN3pin,0)

GPIO.output(Mains1pin,0)

GPIO.output(Mains2pin,0)

door_flag =1

elif count ==1:

if door_flag == 1:

door_flag =0

GPIO.output(DOORpin,1)

time.sleep(1)

GPIO.output(DOORpin,0)

elif count ==2:

GPIO.output(LIGHT1pin,1)

GPIO.output(FAN1pin,1)

elif count ==3:

GPIO.output(LIGHT2pin,1)

GPIO.output(FAN2pin,1)

elif count ==4:

GPIO.output(LIGHT3pin,1)

GPIO.output(FAN3pin,1)

else:

GPIO.output(Mains1pin,1)

GPIO.output(Mains2pin,1)

time.sleep(5)

count =0

elapsed = time.time() - start

if elapsed > 120:	#2 min timeout

print "\nTimeout Occured. Restart Program"

break;

- If CTRL+C is pressed the main loop is broken except KeyboardInterrupt:

RUNNING = False print "\nStopping"

- Actions under 'finally' will always be called finally:

  - Stop and finish cleanly so the pins

  - are available to be used again GPIO.cleanup()

OUTPUT: